DIGITAL MEDIA FUNDAMENTALS

What are Digital Media Fundamentals?

Digital Media Fundamentals refers to the core basic principles that explain how digital information is captured, represented, processed, stored, and displayed.

It is the foundation of understanding:

- How audio becomes MP3

- How images become JPEG

- How videos become MP4

- How computers handle colours, brightness, pixels, and sound waves

Digital Media Fundamentals = the basic building blocks of how digital audio, images, and video work.

Digital Media refers to any form of media that is stored, created, transmitted, or processed using digital (binary) information — meaning 0s and 1s.

Digital media = media represented as digital data.

Examples of Digital Media

Audio: MP3, WAV, AAC, Podcasts
Images: JPEG, PNG, GIF, RAW
Video: MP4, AVI, MOV, YouTube

SIGNALS

A signal is a time-varying physical quantity that is used to convey and transmit information across a physical medium.

Signals fall into two major categories:

- Analogue Signals

- Digital Signals

An analogue signal is a continuous signal that represents information using a smooth and unbroken variation of amplitude over time.
It can take infinite possible values within a range

Examples

Human voice (sound waves), Radio waves (AM/FM)

Key Characteristics

- Continuous in time

- Continuous in amplitude

- Infinite possible values (high resolution)

- Easily affected by noise

- Smooth, natural representation of information

A digital signal is a signal that represents information using discrete values, typically in binary form (0s and 1s).
It is derived by sampling and quantising an analogue signal.

Example MP3 audio files, JPEG/PNG images, and MP4 videos

Key Characteristics

- Discrete in time (sampled)

- Discrete in amplitude (limited levels)

- Represented using binary code

- Less affected by noise

- Easy to process, compress, and store

DIFFERENCE BETWEEN ANALOGUE AND DIGITAL SIGNALS

| Feature, | Analogue Signal, | Digital Signal, |
| --- | --- | --- |
| Nature, | Continuous signal, | Discrete signal, |
| Amplitude Values, | Can take infinite values | Limited, fixed values (binary) |
| Time, | Continuous over time | Exists only at sampled intervals |
| Noise Tolerance, | Low – easily affected by noise | High – resistant to noise and distortion |
| Examples, | Human voice, radio waves, light waves | MP3, JPEG, MP4, digital communication signals |

SAMPLING AND QUANTISATION

Sampling and quantization are the two fundamental processes that convert a continuous analog signal into a digital signal that can be stored, processed, and transmitted by computers. This process is collectively known as Analog-to-Digital Conversion (ADC).

Sampling

Sampling is the process of converting a continuous-time (analogue) signal into a discrete-time signal by measuring its amplitude at regular intervals.

Analogue signals are continuous in time and amplitude.

Computers can only process discrete (digital) values, so we need to capture the signal at specific intervals.

Sampling rate (frequency) is the number of samples taken per second, measured in Hertz (Hz).

Important rule (Nyquist Theorem): The sampling rate must be at least twice the highest frequency in the analogue signal

Example: Sampling a Human Voice

Imagine someone says the word “Hi”.

- The human voice is an analogue signal — it continuously changes in air pressure over time.

- To store it digitally, we need to sample it: measure the amplitude (volume) of the sound at regular time intervals.

- The sampling rate is chosen based on the highest frequency present in the voice.

  - Human speech mostly ranges from 300 Hz to 3,400 Hz (telephone range).

  - According to the Nyquist Theorem, the sampling rate must be at least twice the highest frequency to capture all details.

  - So, 2 × 3,400 Hz ≈ 6,800 Hz → we round up to 8,000 Hz, which is standard for telephone audio.

- At this sampling rate, the system takes 8,000 measurements (samples) per second of the voice’s amplitude.

- Each measurement becomes a sample value, e.g., 0.21, 0.53, 0.78, 0.92.

- Each sample is then stored as a digital number.

Quantization

Refer to the process of converting the continuous range of amplitude values obtained from sampling into a finite set of discrete values (digital numbers).

- Each sample value is still continuous (decimal). Computers cannot store infinite precision.

- Quantisation = rounding each sample to the nearest level determined by bit depth.

Bit depth = the number of bits used to store each sample.

Bit depth examples:

- 4-bit → 16 levels → very rough approximation

- 8-bit → 256 levels → moderate accuracy

- 16-bit → 65,536 levels → very accurate (CD quality)

Higher bit depth → more levels → samples can be stored more accurately → better sound quality

Lower bit depth → fewer levels → more rounding → loss of detail → quantisation noise

Applying Quantisation to Human Voice

Example of sample values from sampling

0.21, 0.53, 0.78, 0.92, …

- Using a 16-bit system, each sample can be rounded to 1 of 65,536 levels.

- This converts the analogue amplitude into a digital number that the computer can store.

Example

| Original Sample | 16-bit Quantised Value |
| --- | --- |
| 0.21 | 0.21001 |
| 0.53 | 0.53001 |
| 0.78 | 0.78001 |
| 0.92 | 0.92001 |

Storing as Digital Media

- After quantisation, all digital numbers (samples) are encoded into a digital file.

- File formats: MP3, WAV, FLAC, etc.

  - Example: MP3 compresses the numbers for smaller size.

  - WAV stores them without compression → highest quality.

- This file is now a digital representation of the voice, which can be:

  - Stored on a computer or smartphone

  - Transmitted over the internet

  - Played back on any digital device

RESOLUTION, QUALITY, AND FILE SIZE

Resolution is a measure of the detail in a digital image or video, defined by the total number of pixels it contains.

1. Spatial Resolution (Images)

- Definition: Number of pixels in width × height (e.g., 1920×1080 for Full HD).

- Impact on Quality: Higher resolution → more detail, sharper edges, smoother appearance.

- Impact on File Size: Higher resolution → more pixels → larger file size.

  - Formula: File Size ∝ Width × Height

2. Temporal Resolution (Video & Audio)

- Video: Measured in Frames Per Second (FPS).

  - Higher FPS → smoother motion, especially for fast action.

  - Higher FPS → larger file size.

- Audio: Measured in Sampling Rate (Hz or kHz, e.g., 44.1 kHz).

  - Higher sampling rate → captures more detail and higher frequencies.

  - Higher sampling rate → larger file size.

COLOUR MODELS

A colour model is a system for representing colours numerically so that computers and devices can process them.

Computers don’t understand “red” or “blue” — they only understand numbers.

- Example: RGB model

  - Red = 255, Green = 0, Blue = 0 → this is “pure red” on a screen.

| Model | Nature | Channels | Typical Use | Key Idea |
| --- | --- | --- | --- | --- |
| RGB | Additive (Light) | Red, Green, Blue | Screens, monitors, TVs, digital cameras | Colours are created by combining light; Black = all off, White = all on |
| CMYK | Subtractive (Ink/Pigment) | Cyan, Magenta, Yellow, Black | Printing on paper | Colours are created by mixing inks; Paper is white, adding ink subtracts light to create colour |
| HSL | Perceptual | Hue, Saturation, Lightness | Graphic design, image editing | Easier for humans to adjust colours intuitively compared to RGB values |

## COLOUR SPACES

## A colour space is a specific mapping of a colour model to a defined set of colours (gamut).

## It defines exactly what each number in a model represents.

Example:

RGB is a model → sRGB is a colour space defining exact red, green, blue shades for screens.

sRGB is a specific colour space → defines exactly what “pure red” or “pure blue” means on your monitor

TRADE-OFFS IN DIGITAL MEDIA: QUALITY, STORAGE, AND BANDWIDTH

When creating, storing, or transmitting digital media (images, audio, video), there is always a trade-off among three key factors:

- Quality – How good the media looks or sounds

- Storage – How much space the media file takes

- Bandwidth – How much data is required to transmit the media over a network

You can usually optimize only two at a time, which forces compromise on the third.

1. Quality vs. Storage

- To increase quality:

  - Higher resolution (more pixels)

  - Higher bit depth (more colours or audio precision)

- Trade-off: File size increases significantly

- Solution: Compression

  - Lossless Compression: Reduces size without losing data (e.g., PNG, FLAC)

  - Lossy Compression: Reduces size by removing “less noticeable” data (e.g., JPEG, MP3, H.264)

- Example: Uncompressed 4K video → extremely large file, difficult to store

2. Storage vs. Bandwidth

- Smaller files → require less storage and less bandwidth for transmission

- Trade-off: High-quality or uncompressed media → large files → higher bandwidth required for streaming

- Example: Video streaming platforms compress videos for smooth delivery over limited bandwidth

3. Bandwidth vs. Quality

- High-quality streaming (e.g., 4K HDR video) → requires high bitrate → high bandwidth

- Low bandwidth connections → must reduce bitrate → apply lossy compression or reduce resolution → lowers quality

- Example:

  - H.265/HEVC codec → same visual quality as H.264 but smaller file → lower bandwidth required

  - Trade-off: Encoding/decoding may need more processing power