PEER-TO-PEER SYSTEMS QUESTIONS

1. A university wants students to share large research datasets directly without using a central server. Describe how a peer-to-peer system can be designed to support this requirement.

-A pure decentralized P2P architecture like a Distributed Hash Table (DHT) can be used. Each dataset is split into chunks, hashed, and assigned to specific peers in the network. Peers selforganize into an overlay network (e.g., using Chord, Kademlia). When a student requests a file, the DHT routes the request to the peer storing the relevant chunk. Redundancy is achieved by replicating chunks across multiple peers. This avoids single points of failure and distributes storage and bandwidth load.

2. A music-sharing application allows users to download files from multiple peers at the same time. Explain how network programming supports this P2P architecture.

-Network programming enables parallel socket connections from one peer to multiple others. Using protocols like BitTorrent, the file is divided into pieces. The client requests different pieces from different peers simultaneously, maximizing download speed. Network programming handles:

- Peer discovery via trackers or DHT.

- Piece selection algorithms (rarest-first).

- Data verification via hash checks.

- Connection management (NAT traversal using UPnP/STUN).

3. A disaster recovery team uses laptops to exchange data when the internet is unavailable. How would a peer-to-peer system be designed for this scenario?

-A mobile ad-hoc network (MANET) P2P system using Wi-Fi Direct or Bluetooth mesh can be deployed. Peers form a dynamic network without infrastructure. A delay-tolerant networking (DTN) approach can store and forward data when connections are intermittent. Data can be replicated across multiple laptops for availability. Services like local service discovery (mDNS/Bonjour) help locate peers.

4. A company wants to reduce server load by allowing clients to share updates among themselves. Describe a suitable P2P design approach.

-A hybrid P2P/CDN approach can be used. When a client receives an update, it caches it and becomes a source for other clients. A central tracker or rendezvous server coordinates initial peer lists. Protocols like Peer-to-Peer Streaming Protocol (PPSP) or WebRTC for data channels allow direct client transfers. This reduces bandwidth and load on the origin server.

5. In a P2P chat application, peers frequently join and leave the network. Explain how the system maintains communication.

-Using a structured P2P overlay like Chord or Pastry, each peer maintains a routing table of neighbors. On joining, the peer contacts a bootstrap node and integrates into the overlay.

Messages are overlay using the peer’s ID. For churn resistance, the system uses:

- Heartbeats to detect failures.

- Successor lists in DHTs for redundancy.

- Cached message queues for offline peers (if supported).

TOPIC: WEB SERVICES & RMI

6. A banking system uses multiple servers to process customer transactions. Explain how Java RMI can be used to invoke remote services.

-Java RMI allows objects in one JVM to invoke methods on objects in another JVM. In a banking system:

- A remote interface BankService defines methods like transferFunds().

- The server implements this interface and registers it with the RMI registry.

- Clients look up the remote object stub from the registry and call methods.

- RMI handles serialization, network communication, and marshalling.

For fault tolerance, multiple RMI servers can be deployed with load balancing.

7. A logistics company exposes shipment data through a Python-based web service. Describe how distributed applications can consume this service.

-Using RESTful APIs or SOAP with Python frameworks like Flask or FastAPI. The service exposes endpoints like /shipments/{id} over HTTP.

Clients (in any language) send HTTP requests (GET/POST) with proper authentication (API keys/OAuth). Data is exchanged in JSON/XML format. Python clients can use requests library; Java clients can use HttpClient; JavaScript clients can use fetch().

8. An online learning platform uses different programming languages across servers. Explain how web services enable interoperability.

-Web services use platform-independent protocols (HTTP, SOAP, REST) and data formats (XML, JSON).

- SOAP: Uses WSDL to describe services, enabling automatic stub generation in any language.

- REST: Relies on HTTP methods; any language with HTTP support can consume it.

- This allows a Java backend, Python analytics service, and JavaScript frontend to communicate seamlessly.

9. A healthcare system requires remote access to patient records. Describe how RMI or Python web services support this functionality.

-Java RMI: Encapsulates patient record access in remote objects; clients call methods like getPatientRecord(). Secure with RMI over SSL.

Python Web Services: Expose endpoints like /api/patient with Flask/Django; use HTTPS, tokenbased authentication, and HL7/FHIR standards for data format. Both allow remote authorized access while abstracting network details.

10. A weather monitoring system collects data from remote stations. Explain how web services help integrate the distributed components.

-Each weather station can host a web service that publishes data via REST/HTTP.

A central aggregation service periodically polls stations or uses webhooks to push data.

Web services enable:

- Loose coupling between stations and central server.

- Scalability (easy to add new stations).

- Standard data format (JSON/XML) for interoperability.

CLOCK SYNCHRONIZATION IN PARALLEL PROCESSING QUESTIONS

- Multiple processors share a single database and must avoid conflicts. Explain how clock synchronization helps manage resource sharing.

-Clock synchronization ensures that all processors have a consistent notion of time. For database access:

- Timestamp ordering can sequence transactions.

- Lock requests can be prioritized by timestamp.

- Concurrency control algorithms like Lamport’s logical clocks or vector clocks can order events to prevent conflicts like dirty reads or lost updates.

- A distributed printing system schedules print jobs from different computers. Describe how synchronized clocks improve fairness.

-With synchronized clocks, each print job can be timestamped when submitted.

A central print scheduler can order jobs by timestamp, ensuring FIFO fairness across all computers. Without synchronization, clocks on different machines might skew, causing jobs from a fast-clock machine to get undue priority.

- A cloud computing platform allocates CPU time to parallel tasks. Explain the role of clock synchronization.

-Clock synchronization helps in:

  - Scheduling decisions: Allocate CPU slices based on global time intervals.

  - Deadline management: Ensures tasks finish before globally agreed deadlines.

  - Logging & auditing: Timestamps in logs from different VMs are consistent for debugging and billing.

- A financial trading system needs accurate timestamps across servers. How does clock synchronization improve reliability?

-Financial trades must be ordered correctly to ensure compliance (e.g., SEC Rule 613).

Network Time Protocol (NTP) or Precision Time Protocol (PTP) synchronizes clocks to microsecond accuracy. This prevents issues like:

  - Out-of-order transactions.

  - Disputes over trade sequence.

  - Regulatory violations.

- A distributed simulation runs tasks in parallel. Explain how synchronized clocks coordinate processing.

-In parallel discrete event simulation, events must be processed in timestamp order.

Synchronized clocks allow:

  - Global virtual time advancement.

  - Synchronization barriers at specific time points.

  - Causality preservation across different simulation nodes.

EVENT ORDERING – LOGICAL & VECTOR CLOCKS QUESTIONS

- A distributed chat system must display messages in the correct order. Explain how logical clocks help achieve this.

-Each message is tagged with a Lamport timestamp. When a node sends a message, it increments its logical clock and attaches the timestamp. Receivers merge timestamps by taking max(local, received) + 1. Messages are ordered by these logical timestamps before display, preserving causal order.

- In a collaborative document editor, multiple users edit the same file. Describe how vector clocks detect conflicting updates.

-Each user’s client maintains a vector clock – an array of counters, one per site.

When a user edits, they increment their own counter and send the vector with the edit.

On receiving edits, the system compares vectors:

  - If one vector is strictly greater, the edit is newer.

  - If vectors are incomparable, a conflict has occurred (e.g., simultaneous edits).

Conflicts are flagged for manual resolution.

- A transaction processing system needs to determine which event happened first. Explain the use of logical clocks.

-Logical clocks (Lamport) provide a partial ordering of events. Each transaction event gets a timestamp. If event A happens-before B, then L(A) < L(B).

This helps in:

  - Serializability – ordering transactions.

  - Deadlock detection – ordering resource requests.

  - Recovery – replaying logs in order.

- A distributed debugging tool tracks event causality. How do vector clocks assist in this scenario?

-Vector clocks track causal dependencies across processes. Each process maintains a vector of counters.

On an event, it increments its own counter.

During debugging, the tool can:

- Reconstruct causal histories.

- Identify if an event at one process could have influenced another.

- Detect race conditions by analyzing vector comparisons.

- A social media platform processes likes and comments across servers. Explain how event ordering is maintained.

- Using a hybrid logical clock (HLC) or vector clocks to timestamp interactions.

- Events (like, comment) are sent to a message queue (Kafka) with timestamps.

- Consumers order events by timestamp for consistent timeline display.

This ensures that a comment posted after a like is not displayed before it.

CRYPTOGRAPHY & SECURITY IN DISTRIBUTED SYSTEMS QUESTIONS

- An e-commerce system transmits sensitive customer data. Explain how encryption secures distributed communication.

- Transport Layer Security (TLS) encrypts data in transit between client and server.

- Sensitive data (credit card, personal info) is encrypted end-to-end.

- At rest, data is encrypted using AES. Keys are managed via a Key Management Service (KMS).

- Encryption ensures confidentiality and integrity.

- A distributed voting system must prevent vote tampering. Describe the role of cryptography.

Cryptography provides:

  - End-to-end encryption of votes.

  - Digital signatures to authenticate voters.

  - Homomorphic encryption for tallying without decrypting votes.

  - Hash chains to ensure votes are not altered (immutable ledger).

- A file-sharing system requires authentication of users. Explain how security techniques ensure authorized access.

  - Public Key Infrastructure (PKI) for user certificates.

  - OAuth 2.0 / OpenID Connect for federated identity.

  - Access Control Lists (ACLs) on files.

  - Challenge-response protocols (e.g., SSH keys) for authentication.

- A cloud-based payroll system stores confidential data. Describe how cryptography protects stored and transmitted data.

  - In transit: TLS 1.3 for data between clients and cloud.

  - At rest: AES-256 encryption of database fields or whole disks.

  - Key management: Cloud KMS (AWS KMS, Azure Key Vault) for encryption keys.

  - Tokenization for sensitive fields like bank account numbers.

- A distributed system is vulnerable to man-in-the-middle attacks. Explain security measures to prevent this.

  - Mutual TLS (mTLS) for both client and server authentication.

  - Certificate pinning to prevent rogue CA certificates.

  - Diffie–Hellman key exchange for forward secrecy.

  - Network segmentation and intrusion detection systems.

OS SERVICES FOR MULTIPLE JOBS QUESTIONS

- A server processes payroll, backups, and reporting tasks simultaneously. Explain how OS services manage multiple jobs.

The OS uses:

  - Process scheduling (e.g., priority-based) to allocate CPU.

  - I/O scheduling to manage disk access for backups.

  - Memory management to isolate tasks.

  - Inter-process communication for coordination.

- A research lab runs simulations in parallel. Describe how the operating system schedules these jobs.

The OS uses parallel job scheduling:

- Multi-core scheduling assigns threads to different cores.

- Gang scheduling for tightly coupled parallel tasks.

- Load balancing across CPUs.

- Fair-share scheduling to prevent starvation.

- A web server handles many client requests at once. Explain how OS multitasking supports this.

The OS provides:

  - Multi-threading – each request can be a thread.

  - I/O multiplexing (epoll/kqueue) for efficient network I/O.

  - Preemptive scheduling to switch between requests.

  - Connection pooling managed by OS network stack.

- A data centre executes batch jobs overnight. How does the OS manage job execution?

Using batch job schedulers (like cron, SLURM, PBS):

  - Job queues prioritized by policies.

  - Resource allocation (CPU, memory).

  - Checkpointing for long jobs.

  - Logging and notification on completion/failure.

- A student computer lab runs multiple applications concurrently. Explain the OS role in resource allocation.

The OS:

  - Time-shares CPU among applications.

  - Allocates memory per process.

  - Manages GPU access if needed.

  - Provides disk quotas to prevent one user from hogging space.

  - Network bandwidth shaping.

JOB SUBMISSION LIBRARIES & UTILITIES QUESTION

- A computing cluster uses job submission libraries. Explain how these libraries support distributed processing.

Libraries like MPI, SLURM, or Hadoop YARN provide APIs to:

  - Submit jobs to a cluster.

  - Handle data distribution.

  - Manage task dependencies.

  - Collect results.

- A weather forecasting system submits analysis jobs automatically. Describe how OS utilities assist.

-OS utilities like cron, systemd timers, or at schedule jobs.

-Scripts invoke cluster submission tools (e.g., sbatch for SLURM).

-Logs are managed via syslog or journald.

- A research institution uses scripts to submit multiple jobs. Explain how libraries simplify job management.

Libraries (e.g., subprocess in Python, Joblib) automate:

  - Job array submission for parameter sweeps.

  - Retry logic on failure.

  - Output collection and parsing.

- A cloud platform allows users to submit workloads remotely. How do OS utilities support this?

OS provides:

  - SSH for remote login and job submission.

  - API daemons (e.g., Docker daemon, Kubelet) to accept remote requests.

  - Resource managers (cgroups) to isolate workloads.

- A university HPC system supports job queues. Explain the role of software libraries.

Libraries like SLURM API, DRMAA, or Celery allow users to:

  - Programmatically submit jobs.

  - Query job status.

  - Cancel or modify jobs.

  - Handle job dependencies in workflows.

PROCESSOR INTER-COMMUNICATION QUESTIONS

- A parallel image-processing application splits tasks across processors. Explain how processors communicate.

Using message passing (MPI) or shared memory:

  - MPI_Send/MPI_Recv to exchange image tiles.

  - Collective operations (MPI_Gather) to combine results.

  - Synchronization barriers after each stage.

- A distributed database synchronizes updates across CPUs. Describe processor intercommunication.

Processors communicate via:

  - Atomic broadcast protocols to propagate updates.

  - Two-phase commit for transactions.

  - Gossip protocols for eventual consistency.

  - RDMA for low-latency sync in clustered databases.

- A scientific application shares intermediate results between processors. Explain how communication is managed.

Using MPI with:

  - Non-blocking sends/receives to overlap computation and communication.

  - Collective operations like MPI Allreduce for global sums.

  - Topology-aware communication to minimize latency.

- A multiplayer game server distributes tasks across cores. How do processors coordinate?

Using thread pools and task queues:

  - One core handles network I/O.

  - Others process game logic.

  - Lock-free data structures for shared state (player positions).

  - Event-driven messaging between cores.

- A parallel search algorithm requires frequent data exchange. Explain inter-processor communication.

Processors exchange boundary data via:

  - Message passing (MPI) for distributed memory.

  - Shared memory with mutexes for multi-core.

  - Work-stealing queues for load balancing.

NAME SERVICES QUESTIONS

- A client application locates services using DNS. Explain how name services support this process.

-DNS translates human-readable names (e.g., service.example.com) to IP addresses.

-Clients query DNS servers recursively/iteratively.

-SRV records can specify port numbers for services.

- A Java RMI application uses an RMI Registry. Describe how it helps clients locate remote objects.

-RMI Registry is a naming service that maps string names to remote object references.

-Server binds objects to the registry.

-Clients look up by name to get stub, then invoke methods.

- An organization uses LDAP to manage user accounts. Explain its role in distributed systems.

LDAP provides a centralized directory service for:

  - User authentication (bind operation).

  - Storing user attributes (email, group).

  - Single sign-on (SSO) across distributed apps.

- A distributed file system allows users to access files transparently. Explain how naming is handled.

Using a global namespace (e.g., in NFS, AFS, or HDFS):

  - Files are named with pathnames independent of physical location.

  - Mount points map local directories to remote shares.

  - Metadata servers manage the namespace.

- A cloud service relies on name resolution for scalability. Describe the importance of name services.

Name services (DNS, service discovery like Consul) allow:

  - Load balancing via round-robin DNS.

  - Failover by updating DNS records.

  - Microservices discovery without hardcoded IPs.

  - Geographic routing via GeoDNS.

CLIENT-SERVER ARCHITECTURE & PARALLEL PROCESSING QUESTIONS

- A client submits tasks to a server for parallel execution. Explain how client–server architecture supports this.

-Client sends tasks over network (RPC/REST).

-Server has a task queue and worker pool.

-Workers execute tasks in parallel, return results to client.

-Examples: HTTP server with background workers, RMI server.

- A web application processes requests using multiple backend servers. Describe the parallel processing design.

-Load balancer distributes requests to backend servers (round-robin, least connections).

-Each server processes requests independently.

-Shared session storage (Redis) maintains state.

-Database connection pooling for parallel DB access.

- A student registration system uses a centralized server. Explain the client–server interaction.

Clients (browsers) send HTTP requests to the server.

-Server processes registration logic, updates database.

-Responses sent back to clients.

-Synchronization via database transactions to prevent double registration.

- A data analytics system distributes workloads from clients to servers. Explain the architecture.

Master-worker architecture:

  - Client submits job to master.

  - Master partitions data, assigns to worker servers.

  - Workers process in parallel, return results to master.

  - Master aggregates and returns to client.

Used in Hadoop MapReduce.

- A banking system processes transactions from many clients simultaneously. Describe how client–server architecture ensures efficiency.

  - Multi-threaded server handles concurrent connections.

  - Connection pooling to database.

  - Transaction pooling to manage DB locks.

  - Load balancing across multiple application servers.

  - Caching (Redis) for frequently accessed data.